ABSTRACT

This project demonstrates a simple home automation system which contains a remote mobile host controller and several home appliances. The home appliances communicate with the host controller through a wireless device such as a Bluetooth enabled mobile phone, in this case, an android based Smart phone. Bluetooth has a range of 10-15 meters, so that we can switch On and Off any electronic appliances within the range. Here the microcontroller can turn On and Off the home appliances accordingly.8051  Microcontroller  is  used  for  controlling  the  whole  process of  this  project. An android application should be installed in user’s mobile or tablet to control the electrical loads. Using this android application user can send the commands to the Bluetooth module to control the electrical loads. Home  appliances  will  be  turned  On  and  Off  when  user  touch  the  button  in  the  Bluetooth  mobile  app  in  Android  mobile  phone. The program which is written to the 8051 microcontroller communicates with Bluetooth module serially to receive the commands. When the user touch any button in Bluetooth controller app then Android phone sends a value to Bluetooth module, after receiving this value, Bluetooth module sends the received value to the microcontroller and then microcontroller reads it and compare it with predefined value. Microcontroller switches the electrical loads automatically based on the commands received from the Bluetooth. If any match is occurred then microcontroller performs relative operation. Same operation will performed each time when button pressed. Operating conventional wall switches is difficult for physically handicapped or elder people. This project provides the solution to this problem by integrating all the electrical appliances to a control unit that can be operated by an Android application device (Android smart phone or Tablet). It  helps  in  creating  a whole  system  for  the  house  and can  connect  all  the  AC  appliances  to  the  8051  microprocessor  using Relays.  Also the project is feasible because the cost of the project is very less as compared to the expensive Wi-Fi based home control systems presently available in the market which require an additional cost of internet services.

LIST OF FIGURES

Fig no.				Description 				    Page no.

- Pin diagram of AT89C51				6

- HC-05 Bluetooth module				9

- IC 7805 pin diagram					10

- 5V relay module						11

- Pin diagram of ULN2003A 				12

- Block diagram 						18

- Circuit diagram						19

- Results							24

CHAPTER 1

INTRODUCTION

Bluetooth  Controlled   Home  Automation  System  using  8051 Microcontroller  is  a  wireless  Bluetooth  technology  to  control  the  Home  Electronic  Appliances  through  an  Android  Phone.

In  past  few  years  there  have  been  so  many  inventions  in  the  field  of  consumer  electronics  such  as  cellular  phone,  air  conditions,  home  security  devices  and  home  theatres.  All  these  appliances  can  be  easily  controlled  by  a  single  controller,  using  personal  area  network  in  a  home  environment.  And  the  market  is  going  towards  the  home  automation  and  networking  and  Bluetooth  is  an  ideal  solution  for  this  purpose.

This  report  demonstrates  a  simple  home  automation  system  which  consists  of  remote  mobile,  host  controller,  and  several  home  appliances.  The  client  module  can  communicate  with  host  controller  through  a  wireless  device,  in  this  case  a  Bluetooth  enabled  mobile  phone.

Presently,  conventional  wall  switches  located  in  different  parts  of  the  house  makes  it  difficult  for  the  user  to  go  near  them  to  operate.  It  becomes  even  more  difficult  for  the  elderly  or  physically  handicapped  people  to  do  so.  Remote  controlled  home  automation  system  provides  a  simpler  solution  with  Android  application  technology.

Home  automation  is  not  a  new  thing  but  the  most  advanced  home  automation  systems  in  existence  today  require  a  big  and  expensive  infrastructure.  This  means  that  it  often  is  not  feasible  to  install  a  home  automation  system  in  an  existing  building.  This  project  however,  proposes  a  low  cost  and  flexible   automation  system  that  can  easily  control  lights,  fans,  and  TV  from  an  android  based  mobile  phone.  Here  HC-05  Bluetooth  module  and  8-bit  microcontroller  8051  are  used  for  switching.

CHAPTER 2

PROJECT BRIEF

Home  automation  system  using  microcontroller  8051  and  a  Bluetooth  module  controls  several  electronic  home  appliances  with  the  range  of  Bluetooth,  using  an  android  phone  with  a  Bluetooth  app.

AIM:

The  Project  aims  at  controlling  the  home  electronic  appliances  through  an  Android  phone  using  8051  microcontroller  and  Bluetooth  technology.

OBJECTIVES:

- To  have  a  basic  idea  about  microcontroller  based  home  automation  system.

- To  get  a  basic  idea  about  Bluetooth  module  and  their  working.

- To  get  an  idea  about  interfacing  of  Bluetooth  module  with  8051  microcontrollers.

PROJECT RELEVENCE:

This  project  finds  application  and  relevance  where  one  desires  to  control  the  basic  home  systems  and  appliances   using  his  smart  phone  and  to  access  them  from  remote  point.  Sometimes  we  find  it  difficult  to  switch  ON  and  OFF  the  devices  when  the  switches  are  located  far  away  from  where  we  sit.  And  the  condition  is  even  worse  for  the  elderly  and  those  who  are  handicapped.  With  home  automation,  convenient  control  of  our  home  is  at  our  fingertips.  And  using  a  Bluetooth  technology  rather  than  a  Wi-Fi  based  system  reduces  the  overall  cost  of  the  system  making  it  available  to  the  common  people.

CHAPTER 3

DESCRIPTION

3.1 MICROCONTROLLER – AT89C51

DESCRIPTION

The AT89C51 is a low-power, high-performance CMOS 8-bit microcomputer with 4 Kbytes of Flash Programmable and Erasable Read Only Memory (PEROM). The device is manufactured using Atmel’s high density nonvolatile memory technology and is compatible with the industry standard MCS-51 instruction set and pinout.

The on-chip Flash allows the program memory to be reprogrammed in-system or by a conventional nonvolatile memory programmer. By combining a versatile 8-bit CPU with Flash on a monolithic chip, the Atmel AT89C51 is a powerful microcomputer which provides a highly flexible and cost effective solution to many embedded control applications.

The AT89C51 provides the following standard features: 4Kbytes of Flash, 128 bytes of RAM, 32 I/O lines, two 16-bit timer/counters, a five vector two-level interrupt architecture, a full duplex serial port, on-chip oscillator and clock circuitry. In addition, the AT89C51 is designed with static  logic for operation down to zero frequency and supports two software selectable power saving modes.

Fig 1 – Pin  Diagram of  AT89C51  Microcontroller.

PIN   DESCRIPTION

Pins 1-8: Port 1

Each of these pins can be configured as an input or an output.

Pin 9: RESET

A logic one on this pin disables the microcontroller and clears the contents of most registers. In other words, the positive voltage on this pin resets the microcontroller. By applying logic zero to this pin, the program starts execution from the beginning.

Pins10-17: Port 3

Similar to port 1, each of these pins can serve as general input or output. Besides, all of them have alternative functions.

Pin 10: RXD

Serial asynchronous communication input or Serial synchronous communication output.

Pin 11: TXD

Serial  asynchronous communication output or Serial synchronous communication clock output.

Pin 12: INT0

Interrupt 0 input.

Pin 13: INT1

Interrupt 1 input.

Pin 14: T0

Timer 0 clock input.

Pin 15: T1

Timer 1 clock input.

Pin 16: WR

Write to external (additional) RAM.

Pin 17: RD

Read from external RAM.

Pin 18, 19:  X2, X1

Internal oscillator input and output. A quartz crystal which specifies operating frequency is usually connected to these pins. Instead of it, miniature ceramics resonators can also be used for frequency stability. Later versions of microcontrollers operate at a frequency of 0 Hz up to over 50 Hz.

Pin 20: GND

Ground.

Pin 21-28: Port 2

If there is no intention to use external memory then these port pins are configured as general inputs/outputs. In case external memory is used, the higher address byte, i.e. addresses A8-A15 will appear on this port. Even though memory with capacity of 64Kb is not used, which means that not all eight port bits are used for its addressing, the rest of them are not available as inputs/outputs.

Pin 29: PSEN (Program Store Enable)

If external ROM is used for storing program then a logic zero (0) appears on it every time the microcontroller reads a byte from memory.

Pin 30: ALE (Address Latch Enable)

Prior to reading from external memory, the microcontroller puts the lower address byte (A0-A7) on P0 and activates the ALE output. After receiving signal from the ALE pin, the external register (usually 74HCT373 or 74HCT375 add-on chip) memorizes the state of P0 and uses it as a memory chip address. Immediately after that, the ALU pin is returned its previous logic state and P0 is now used as a Data Bus. As seen, port data multiplexing is performed by means of only one additional (and cheap) integrated circuit. In other words, this port is used for both data and address transmission.

Pin 31: EA  (External Access)

By applying logic zero to this pin, P2 and P3 are used for data and address transmission with no regard to whether there is internal memory or not. It means that even there is a program written to the microcontroller, it will not be executed. Instead, the program written to external ROM will be executed. By applying logic one to the EA pin, the microcontroller will use both memories, first internal then external (if exists).

Pin 32-39: Port 0

Similar to P2, if external memory is not used, these pins can be used as general inputs/outputs. Otherwise, P0 is configured as address output (A0-A7) when the ALE pin is driven high (1) or as data output (Data Bus) when the ALE pin is driven low (0).

Pin 40: VCC

+5V  power supply.

3.2 HC-05  BLUETOOTH  MODULE

HC-05 Bluetooth module consists two things, one is Bluetooth serial interface module and a Bluetooth adaptor. Bluetooth serial module is used for converting serial port to Bluetooth. Default baud rate of new Bluetooth module is 9600 bps. To  operate, connect Rx and Tx to controller or serial converter and give 5 volt dc regulated power supply to module. Bluetooth module has two modes one is master mode and second one is slave mode. User can set either mode by using some AT commands.

DIAGRAM

Fig 2 – HC-05 bluetooth module

PIN  DESCRIPTION

- STATE   → Open

- Rx           → Serial receiving pin

- Tx           → Serial transmitting pin

- GND       → ground

- Vcc         → +5volt dc

- EN          → to enter in AT mod

3.3 IC 7805

7805 is a voltage regulator integrated circuit. It is a member of 78xx series of fixed linear voltage regulator ICs. The voltage source in a circuit may have fluctuations and would not give the fixed voltage output. The voltage regulator IC maintains the output voltage at a constant value. The xx in 78xx indicates the fixed output voltage it is designed to provide. 7805 provides +5V regulated power supply. Capacitors of suitable values can be connected at input and output pins depending upon the respective voltage levels.

Fig 3- IC7805 pin diagram

PIN  DESCRIPTION

| PIN NO | PIN | Function | DESCRIPTION |
| --- | --- | --- | --- |
| 1 | INPUT | Input voltage (7V-35V) | In this pin of the IC positive unregulated voltage is given in regulation. |
| 2 | GROUND | Ground (0V) | In this pin where the ground is given. This pin is neutral for equally the input and output. |
| 3 | OUTPUT | Regulated output; 5V (4.8V-5.2V) | The output of the regulated 5V volt is taken out at this pin of the IC regulator. |

3.4  5V RELAY MODULE

It has three pins, the VCC, GND and Signal. It can act as switch if the circuit and the load circuit have different supply voltage. It is commonly use if the load circuit is AC. It is a switch used to connect isolated connection from the circuit using a circuit signal. It has red LED that turns on every time the coil is energized or the signal pin has a high input.

Fig 4 -5V relay module

PIN CONFIGURATION

+      : 5V power supply

-       : Ground

S      : Signal from the Controller

NC   : Normally Closed

NO   : Normally Open

COMMON   : Common

3.5 ULN2003A  RELAY  DRIVER

ULN2003 is a high voltage and high current Darlington array IC. It contains seven open collector darlington pairs with common emitters. A darlington pair is an arrangement of two bipolar transistors.

ULN2003 belongs to the family of ULN200X series of ICs. Different versions of this family interface to different logic families. ULN2003 is for 5V TTL, CMOS logic devices. These ICs are used when driving a wide range of loads and are used as relay drivers, display drivers, line drivers etc. ULN2003 is also commonly used while driving Stepper Motors.

Each channel or darlington pair in ULN2003 is rated at 500mA and can withstand peak current of 600mA. The inputs and outputs are provided opposite to each other in the pin layout. Each driver also contains a suppression diode to dissipate voltage spikes while driving inductive loads.

Fig 5 – pin diagram of ULN2003A

PIN  DESCRIPTION

| Pin No | Function | Name |
| --- | --- | --- |
| 1 | Input for 1st channel | Input 1 |
| 2 | Input for 2nd channel | Input 2 |
| 3 | Input for 3rd channel | Input 3 |
| 4 | Input for 4th channel | Input 4 |
| 5 | Input for 5th channel | Input 5 |
| 6 | Input for 6th channel | Input 6 |
| 7 | Input for 7th channel | Input 7 |
| 8 | Ground (0V) | Ground |
| 9 | Common free wheeling diodes | Common |
| 10 | Output for 7th channel | Output 7 |
| 11 | Output for 6th channel | Output 6 |
| 12 | Output for 5th channel | Output 5 |
| 13 | Output for 4th channel | Output 4 |
| 14 | Output for 3rd channel | Output 3 |
| 15 | Output for 2nd channel | Output 2 |
| 16 | Output for 1st channel | Output 1 |

CHAPTER 4

SERIAL DATA TRANSMISSION AND RECEPTION

Serial Interface

The serial port of 8051 is full duplex, i.e., it can transmit and receive simultaneously. The register SBUF is used to hold the data. The special function register SBUF is physically two registers. One is, write-only and is used to hold data to be transmitted out of the 8051 via TXD. The other is, read-only and holds the received data from external sources via RXD. Both mutually exclusive registers have the same address 099H.

Serial Port Control Register (SCON)

Register SCON controls serial data communication. Address: 098H (Bit addressable)

Mode select bits

SM2: multi processor communication bit REN: Receive enable bit TB8: Transmitted bit 8 (Normally we have 0-7 bits transmitted/received) RB8: Received bit 8 TI: Transmit interrupt flag RI: Receive interrupt flag

Power Mode control Register

Register PCON controls processor powerdown, sleep modes and serial data bandrate. Only one bit of PCON is used with respect to serial communication. The seventh bit (b7)(SMOD) is used to generate the baud rate of serial communication.

Address: 87H

SMOD: Serial baud rate modify bit GF1: General purpose user flag bit 1 GF0: General purpose user flag bit 0 PD: Power down bit IDL: Idle mode bit

Data Transmission

Transmission of serial data begins at any time when data is written to SBUF. TI is set to 1 when data has been transmitted. This signifies that SBUF is empty so that another byte can be sent.

Data Reception

Reception of serial data begins if the receive enable bit is set to 1 for all modes. Receive interrupt flag, RI, is set after the data has been received in all modes. The data gets stored in SBUF register from where it can be read.

Serial Data Transmission Modes:

Mode-0: In this mode, the serial port works like a shift register and the data transmission works synchronously with a clock frequency of fosc /12. Serial data is received and transmitted through RXD. 8 bits are transmitted/ received at a time. Pin TXD outputs the shift clock pulses of frequency fosc /12, which is connected to the external circuitry for synchronization. The shift frequency or baud rate is always 1/12 of the oscillator frequency.

Mode-1 (standard UART mode) :

In mode-1, the serial port functions as a standard Universal Asynchronous Receiver Transmitter (UART) mode. 10 bits are transmitted through TXD or received through RXD. The 10 bits consist of one start bit (which is usually '0'), 8 data bits (LSB is sent first/received first), and a stop bit (which is usually '1'). Once received, the stop bit goes into RB8 in the special function register SCON. The baud rate is variable. Bit time= 1/fbaud. In receiving mode, data bits are shifted into the receiver at the programmed baud rate. The data word (8-bits) will be loaded to SBUF if RI is zero and mode bit SM2 is zero or stop bit is 1. After the data is received and the data byte has been loaded into SBUF, RI becomes one.

CHAPTER 5

BLOCK DIAGRAM AND WORKING EXPLANATION

In this project we have used 8051 microcontroller for controlling the whole process of this project. And a Bluetooth module is used for controlling the home appliances wirelessly. Home appliances will turned ON and OFF when user will touch button in the Bluetooth mobile app in Android mobile phone. To run this project, first we need to download Bluetooth app form Google play store. We can use any Bluetooth app that can send data using Bluetooth. Here are some apps name that can be used: - Bluetooth Spp pro - Bluetooth controller After installing the App, you need to open it and then search Bluetooth device and select HC-05 Bluetooth device. And then configure keys. Here in this project we have used Bluetooth controller app. 1.Download and install Bluetooth Controller. 2.Turn ON mobile Bluetooth. 3.Now open Bluetooth controller app 4.Press scan 5.Select desired Bluetooth device (Bluetooth Module HC-05). 6.Now set keys by pressing set buttons on screen. To set keys we need to press ‘set button’ and set key as given below: Button; Data; Operation. Fan On; 1; Fan Turned On. Fan Off; 2; Fan Turned Off. Light On; 3; Light Turned On. Light Off; 4; Light Turned Off. TV On; 5; TV Turned On. TV Off; 6; TV Turned Off.

After setting keys press ok. You can see that there are 9 buttons in which first row is for fan controlling, second one is for light controlling and last one is for TV controlling. Means First row’s ON and OFF buttons are used to ON and OFF the fan, second row’s buttons are for Light and third ones are for TV. We have used three bulbs of different colors instead of TV and fan, for demonstration purpose. Now, when we touch any button in Bluetooth controller app then Android phone sends a value to Bluetooth module, after receiving this value, Bluetooth module sends the received value to the microcontroller and then microcontroller reads it and compare it with predefined value. If any match is occurred then microcontroller performs relative operation. Same operation will performed each time when button pressed. Fig 6 – Block diagram Now, when user touch ‘Fan On’ button in Bluetooth controller app then microcontroller receives ‘1’ via Bluetooth module and then controller Switch ‘On’ the Fan by using relay driver and relay. And when user touch ‘Fan Off’ button in Bluetooth controller app then microcontroller receives ‘2’ via Bluetooth module and then controller Switch ‘Off’ the Fan by using relay driver and relay. Likewise 3,4,5,6 numbers are sent by Android Phone, when Light On, Light Off, TV On, TV Off button has been touched respectively

CHAPTER 6

CIRCUIT DIAGRAM AND EXPLANATION

Circuit connections of this project are very simple. Bluetooth module’s Rx and Tx pins are directly connected to the Tx and Rx pins of Microcontroller. Three 5 volt relays are used as a switch for turning ON and OFF the home appliances running on AC mains. And a relay driver ULN2003A is used for driving relays. Fan, Light and TV are connected at P1.3, P1.4 and P1.5 through relays and relay driver. An 11.0592 MHz crystal oscillator is used in this circuit for generating clock signal for microcontroller. And a 5 volt voltage regulator LM7805 is used for provide 5 volt for the whole circuit.

Fig 6- Circuit diagram

CHAPTER 7

PROGRAM

//including header files and defines input, output pins and variables

#include<reg51.h>

sbit Fan=P1^3;
sbit Light=P1^4;
sbit TV=P1^5;

char str;
 char Charin=0;

//creating a function for delay

void delay(int time)
{
 unsigned int i,j;
 for(i=0;i<time;i++)
 for(j=0;j<1275;j++);
}

//configuring 9600bps baud rate at 11.0592MHz Crystal Frequency

void Serialwrite(char byte)
{
  SBUF=byte;
  while(!TI);
  TI=0;
}

void Serialprintln(char *p)
{
  while(*p)
  {
    Serialwrite(*p);
    p++;
  }
  Serialwrite(0x0d);
}

void Serialbegin()
{
   TMOD=0x20;
   SCON=0x50;
   TH1=0xfd;
   TR1=1;
}

// initialized UART and monitored the SBUF register for receiving the data

// then data is matched and compared with predefined values and relative operation is performed

void main()
{
  P1=0x00;
  Serialbegin();
  Serialprintln("System Ready...");
  delay(50);
  while(1)
  {
    while(!RI);
    Charin=SBUF;
    str=Charin;
    RI=0;
      if(str=='1')
      {
        Fan=1;
        Serialprintln(" Fan ON");
        delay(50);
      }
      else if(str=='2')
      {
        Fan=0;
        Serialprintln(" Fan OFF");
        delay(50);
      }

else if(str=='3')
      {
        Light=1;
        Serialprintln(" Light ON");
        delay(50);
      }

else if(str=='4')
      {
        Light=0;
        Serialprintln(" Light OFF");
        delay(50);
      }

else if(str=='5')
      {
        TV=1;
        Serialprintln(" TV ON");
        delay(50);
      }

else if(str=='6')
      {
        TV=0;
        Serialprintln(" TV OFF");
        delay(50);
      }
      str=0;
  }
}

So that’s how we can create a whole system for the house and can connect all the AC appliances to the 8051 microcontroller using Relays. And this Bluetooth controlled home automation system can be operated from a Smart phone.

CHAPTER 8

COST ESTIMATION

| SL.NO | COMPONENTS | QUANTITY | COST(in Rs.) |
| --- | --- | --- | --- |
| 1. | Atmel 89C51 | 1 | 70/- |
| 2. | Bluetooth module HC-05 | 1 | 350/- |
| 3. | Relay | 1 | 300/- |
| 4. | ULN2003 | 1 | 20/- |
| 5. | Bulb | 3 | 15/- |
| 6. | Holder | 3 | 80/- |
| 7. | Wire | 5 metres | 50/- |
| 8. | IC 7805 | 1 | 85/- |
| 9. | Burning kit | 1 | 1500/- |
| 10. | 10µF capacitor | 1 | 5/- |
| 11. | 1000µF capacitor | 1 | 5/- |
| 12. | 0.1µF capacitor | 1 | 5/- |
| 13. | 47µF capacitor | 1 | 5/- |
| 14. | 22Pf capacitor | 2 | 5/- |
| 15. | 1K resistor | 1 | 5/- |
| 16. | 10K resistor | 1 | 5/- |
| 17. | 230/12V Transformer | 1 | 100/- |
| 18. | Diode 1N4007 | 4 | 4/- |
| 19. | 11.0592 MHz crystal oscillator | 1 | 20/- |
| 20. | LED | 1 | 2/- |
| TOTAL COST |  |  | Rs.3,008/- |

CHAPTER 9

RESULTS

The project “home automation using AT89C51” has been successfully designed and tested. The  project  designed  is  very  practical  in  nature  because  everything  can  be  controlled  with  the  help  of  just  a  mobile  phone  which  is  widely  available  nowadays  and  also  proves  to  be  handy.

With  home  automation,  convenient  control  of  our  home  is  at  our  fingertips.  And  using  a  Bluetooth  technology  rather  than  a  Wi-Fi  based  system  reduces  the  overall  cost  of  the  system  making  it  available  to  the  common  people.

Fig 7 – Results

CONCLUSION AND FUTURE SCOPE

CONCLUSION

The  home  automation  system  has  been  experimentally  proven  to  work  satisfactorily  by  connecting  sample  appliances  to  it  and  the  appliances  were  satisfactorily  controlled  from  a  wireless  mobile  device.  The  Bluetooth  client  was  successfully  tested  on  a  multitude  of  different  mobile  phones  from  different  manufacturers,  thus  proving  its  portability  and  wide  compatibility.

Home  automation  conveys  incredible  convenience  and  comfort  to  its  users.  The  system  being  discussed  in  the  project  is  a  robust  and  easy  to  use  system.  According  to  this  project  all  the  controls  would  be  in  the  hands  of  the  user,  who  is  provided  with  the  facility  of  monitoring  all  the  appliances  within  the  communication  range  through  Bluetooth  and  can  check  the  status  of  the  appliances  at  whatever  time  of  the  day  using  his  phone.  Apart  from  increasing  the  comfort,  the  system  also  increases  the  home’s  energy  efficiency  and  savings  by  remotely  powering  off  systems  and  appliances  when  they  aren’t  in  use.  Hence, home automation literally pays off.

This  project  will  not  only  provide  convenience  to  the  common  man  but  will  be  a  boon  for  the  elderly  and  disabled.

FUTURE SCOPES

- Using a Bluetooth controller app with more keys and increasing the number of relays we can control more number of devices in home.

- Using any other Bluetooth module that supports multi-user interface instead of  HC-05 more than one users can be allowed to control the devices.

- The  project  can  be  further  developed  by  integrating  it  with  the  internet  to  monitor  the  home  while  sitting  in  a  remote  area.  By  doing  this,  one  can  keep  an  eye  on  his  or  her  home  through  internet  connected  to  the  user’s  mobile  phone  or  PC  or  laptop.  This  will  not  only  improve  the  security  of  the  one’s  home  in  this  modern  day  world  but  will  also  assist  in  conservation  of  energy  like  if  one  left  any  home  appliances  switched  on  by  mistake,  then  he  can  check  the  status  of  the  appliance  on  the  graphical  interface  made  on  his  mobile  and  can  switch  it  of  using  the  internet  connectivity.

BIBILIOGRAPHY

1. Muhammed Ali Mazidi-AVR Microcontroller and Embedded Systems

2. Google-Wikipedia

3. https://circuitdigest.com/microcontroller-projects/bluetooth-controlled-home-automation-using-  8051